SECTION 01 55 00

VEHICLE AND PEDESTRIAN CONTROLS

- GENERAL

      - RELATED DOCUMENTS

        - Construction Drawings, Technical Specifications, Addenda, and general provisions of the Contract, including Contract General Conditions and Supplementary General Conditions and other Division 01 Specification Sections, apply to this Section.

      - SECTION INCLUDES

        - Section specifies requirements for construction activities impacting the Campus Community outside the designated construction site, as well as requirements for Contractor ingress to and egress from the project site. Section includes, but is not limited to the following:

          - Construction activities within or adjacent to pedestrian walkways and thoroughfares.

          - Construction within landscape and hardscape areas outside the designated Project site area.

          - Procedures for work within city streets and campus roads.

          - Haul routes and temporary traffic Control.

          - Contractor parking.

      - WORK WITHIN AREAS OF PEDESTRIAN ACCESS

        - General: These requirements apply to all work required on the Campus outside the designated Project Site. Requirements also apply to activities occurring on the Project Site, which impact adjacent areas of the Campus.

        - Fencing of Work Areas

          - All work areas within the campus and public spaces shall be fenced with minimum 8 feet chain link portable fence sections, with 1-1/2” top, bottom and side rails. All fencing shall be covered with blue fabric shade cloth material, secured to top, bottom and side rails with integral metal eyelets. Shade cloth shall not be left unsecured.  Fencing materials shall be maintained in good, damage free condition at all times.

            - Fencing shall extend around and enclose entire work area, as well as stored materials and equipment.

            - Fencing shall be secured in a closed condition when not required to be open to allow completion of the work. Fencing shall be secured each day at the close of work.

            - The use of alternate materials such as barricades, delineators and caution tape to enclose or delineate work areas will not be accepted.

            - 3 sand bags shall be placed on every stand.  Contractor shall replace sand bags whenever a sand bag ruptures.

            - Contractor can tie-back fencing to fixed stakes as required in lieu of sand bags.  Tie backs shall not be trip hazards.

            - Plastic water filled K-rail can be used in lieu of fencing when approved in advance by the University.

        - Sidewalk Closures and Restrictions

          - Use Cal-Trans Standard reflectorized signage where required to indicate closure of sidewalks, temporary revisions to crosswalks and other impacts to normal pedestrian walk routes.

          - Where sidewalks are partially restricted due to construction activities, a minimum width of 48” shall be maintained.

          - Bases for temporary fencing shall not extend into the required walk area.

          - Where portions of a sidewalk are temporarily closed, temporary fencing shall be placed at the nearest intersection to prevent the site impaired from traveling in a direction that will require them to eventually stop and return to said intersection.  Pedestrian detour signs and “sidewalk closed” signs shall also be provided at the point of closure.

        - Access for construction equipment and material deliveries

          - All haul routes and delivery routes shall conform to the routes designated in Contractor’s approved Work Plans. Refer to Section 01 14 00 for requirements.

          - Times for delivery of materials and hauling shall comply with the requirements of the Contract Documents and approved Contractor Work Plans.

          - No staging or parking of vehicles or construction equipment will be allowed outside the Project Site, except within the work areas designated in the approved Contractor Work Plans.

          - Flagman Requirements

            - All major vehicles and equipment using approved haul routes that travel over intercampus pedestrian thoroughfares shall be escorted by at least one flagman until the vehicle or equipment is within the confines of the project site. Contractor is advised that the Campus Community includes a large volume of students and staff with disabilities, including but not limited to wheel chair users, persons with hearing impairments, and persons with sight impairments; for this reason, escorting of equipment and vehicle traffic will be strictly enforced.

              - Flagman shall be trained and shall direct pedestrians and traffic in accordance with the requirements set forth in Article 1.4 below.

            - Entry exit gates to the project site shall be left in a closed position at all times, unless a flagman is stationed at the gate to control unauthorized entry into the project site.

          - Maintenance of Thoroughfares

            - Pedestrian thoroughfares and crossings shall be maintained in a safe, clean condition, free of dirt, gravel and other debris resulting from construction operations at all times.

            - Where work occurs on or adjacent to pedestrian thoroughfares, Contractor shall employ adequate measures (such as sandbagging, earthen barriers, etc.) to ensure that walks are protected from overflow of construction materials or runoff into the pedestrian area.

            - Where work occurs on or adjacent to pedestrian thoroughfares, Contractor shall employ adequate measures to ensure that walks are protected from overhead hazards, such as falling debris. Provide covered walkway structures and other measures as required to comply with O.S.H.A. standards.

            - Contractor shall confirm local Fire Dept. requirements for access to the construction site and other Campus facilities impacted by the Work throughout the course of construction. Where Fire Dept. access must be maintained at specific areas, Contractor shall tailor the Work Plan and provide necessary temporary measures to accommodate requirement.

          - Trenching Operations

            - Where trenching occurs through, across or adjacent to pedestrian thoroughfares, the work shall comply with the approved Work Plan for the area in question.

            - Temporary pedestrian crossings required due to trenching operations:

              - Steel plating shall be placed across trenches and trench bracing shall be installed in accordance with W.A.T.C.H. standards as referenced in section 1.04 below.

              - Minimum 6 feet high chain link fencing sections (per paragraph 1.2-B above) shall be installed inside the edge of the plating on each side to clearly delineate the path of travel and prevent pedestrians from stepping into trench area.

              - All steel plating shall have beveled edges and shall comply with A.D.A. requirements for path of travel. Edges of plates at each approach shall be painted with a 1” safety yellow contrasting band. Plating shall be anchored in accordance with W.A.T.C.H. standards and carry appropriate traffic ratings where it is required to carry emergency response vehicle traffic.

              - The use of barricades, delineators and or caution tape in lieu of the required temporary fencing sections is unacceptable.

      - WORK WITHIN ROADWAYS AND PARKING AREAS

        - General Requirements

          - All construction activities which occur within campus roadways and parking areas shall comply with the 2012 version of the City of Los Angeles Work Area Traffic Control Handbook (W.A.T.C.H.) for traffic control, signage and barricading, as supplemented by these specifications. Where conflicts exist between specific requirements, the more stringent requirement shall apply. W.A.T.C.H. standards are available from Building News Inc., 3055 Overland Ave., Los Angeles, Ca., 90034 - Phone: 310/202-7775.

          - Flagman requirements and operations shall comply with W.A.T.C.H. standards and the State of California, Dept. of Transportation “Instructions to Flaggers” - 2012 Edition.

          - Signage: All temporary traffic control signage shall comply with California Vehicle Code Section 21400 and California Dept. of Transportation (Cal-Trans) standards. All signage shall be reflectorized.

          - Where trenches, excavations or other work is required within streets, the Work shall be scheduled so as to maintain a minimum of one open traffic lane at all times. A minimum of two lanes as required allowing safe 2-way traffic shall be restored prior to completion of Contractor’s operations each day.

          - All work within University roadways and parking areas requires approval of Contractor’s Work Plan prior to commencement. Refer to Section 01 14 00 for Work Restrictions.

        - Fencing, Barricades and Traffic Plating

          - All work areas shall be fenced in compliance with paragraph 1.3-B above. Modifications to this requirement due to specific access requirements for completion of the work shall be requested by Contractor in the Work Plan Submittal for a designated area.

          - Type 1 Barricades as referenced in the W.A.T.C.H. standards are not acceptable for use on the Project. Contractor shall use type II or type III barricades where required.

          - Where temporary traffic controls must remain in place overnight or at other times when Contractor is not continuously present in the work area, cones, plastic delineators and other lightweight traffic control devices subject to displacement shall not be used for traffic control.

          - Where temporary fencing and/or barricades remain in place overnight, Type II barricades with flashing amber lights shall be used to delineate the protruding corners of the of the work area enclosure at the approach from each direction.

          - Where trenches or excavations of a depth of 3’-0” or deeper and a width of 2’-0” or greater are directly adjacent to a drive lane, the trench shall be plated in accordance with W.A.T.C.H. standards, or concrete barricades (k-rail) shall be installed to protect vehicle traffic from entering the excavation during times when the work area is not manned by Contractor.

          - Where trenches or excavations of a depth of 4’-0” or greater are directly adjacent to a drive lane or pedestrian walk, the protective fencing shall be a minimum horizontal distance of 4’-0” from the edge of the excavation.

          - All traffic plates shall be beveled in the direction of vehicle traffic and secured in place. Where work occurs at pedestrian crossings, comply with Article 1.3 above.

          - Comply with W.A.T.C.H. standards for sizing of traffic plates and shoring of trenches up to 4’ in width. For trenches exceeding 4’ in width, Contractor shall engage a Civil Engineer registered in the State of California to design plating and shoring system.

        - Flagman Requirements

          - Whenever existing traffic lanes are altered, contractor shall provide properly equipped and trained flagmen to direct traffic. Comply with W.A.T.C.H. standards and Cal-Trans “Instructions to Flaggers”.

          - Whenever a section of two-way traffic is temporarily reduced to one lane, a minimum of two flagmen shall be provided to ensure proper traffic control in each direction. 2-way radio devices shall be used for communication between the flagmen where both direct line of site and audible communication cannot be maintained.

          - Flagmen shall be dedicated solely to traffic and pedestrian control and shall not perform additional duties while assigned as flagmen.

        - Signage

          - Traffic control signage shall be provided as required for safe and proper direction of vehicles and in accordance with the requirements listed in paragraph 1.4-A-3 above.

          - All signage shall be reflectorized.

          - Temporary traffic control signs shall be California Dept. of Transportation standard type as listed in the following schedule.

- Sign Type	Designation 	Size

Stop	R1R	30 x 30

Speed limit	R2R	24 x 30

Keep right/left	R7R	18 x 24

Do not enter	R11R	24 x 24

No right turn	R16R	24 x 24

No left turn	R17R	24 x 24

No parking	R26DR	12 x 12

No parking/fire lane	R26RFL	12 x 18

No parking anytime	R28R	12 x 18

Yield	R39R	30” triangle

Disabled parking	R99R	12 x 18

Exit only	R108R	18 x 24

Enter only	R109R	18 x 24

No pedestrians 	R96R	18 x 18

Use crosswalk	R96BR	18 x 12

Two-way traffic ahead	R40R	24 x 30

Merge 	WLR (L) or (R)	24 x 24

        - Haul Routes

          - Haul Routes for Construction activities and delivery of materials shall strictly adhere to routes designated in the contract documents. All vehicles and equipment are required to use designated routes only. Deviations from designated haul routes shall only be permitted where previously authorized in Contractor’s approved Work Plans.

          - Continuous or major hauling on campus roads shall be restricted to the hours of 7:00 am through 6:00 p.m. unless otherwise authorized by the University Representative.

          - Contractor shall comply with hauling and truck traffic requirements on all City roads and shall obtain all required permits and authorizations. Weight loads carried by vehicles shall be within capacity recommended by manufacturer and shall comply with applicable laws and regulations relating to allowable capacities for specific roads.

          - Roads shall be maintained in a clean condition at all times. Sweeping of roads shall occur at minimum on a daily basis, or more often as required by continual hauling operations or construction traffic.

          - All loads shall be covered with secured tarpaulins when gravel, asphalt, debris, or other loose materials are removed from or hauled into the Campus.

          - Truck staging shall not occur on any campus road, or City road within the Campus, unless prior authorization is received through approval of the Contractor’s Work Plan.

          - Provide protection against damage to existing sidewalks, curbs and gutters and other improvements at locations where construction vehicles enter. Contractor shall be responsible for repair of all damage resulting from its operations. Damage to concrete shall be repaired by replacement of full sections to the nearest existing construction joint in each direction.

        - Emergency Response Access

          - Contractor shall maintain adequate provisions for passage of emergency response vehicles (ambulances, fire trucks etc.) over campus roads and inner-campus thoroughfares at all times.

          - At all times that work is occurring which requires trenching, excavations, or other blockages of any fire lane or emergency access location, Contractor shall have traffic plating and other materials and equipment on hand as required to permit immediate passage of response vehicles in the case of an emergency. At no time shall said blockages be left unmanned.

      - PARKING CONTROL

        - Contractor and all subcontractors, suppliers, etc. are required to purchase University parking passes.  Contractor Parking Permits are $___/mo [insert amount].  Contractor Parking Passes are required for all vehicles parking on campus regardless of location.

        - Contractor, subcontractors and suppliers shall park within the Construction site and other authorized areas as identified in the Contract Documents.

        - Any parking in University lots or stalls outside the authorized areas identified in the Contract Documents requires payment of the current University parking fees as evidenced by display of a valid CSUN parking permit. Permits may be obtained by purchasing them through the University Dept. of Public Safety.

        - Contractor, subcontractors and suppliers shall at no time park any vehicle on the inner-campus, outside the confines of the construction site as designated in the Contract Documents. Vehicles in non-compliance will be cited and towed.

- PRODUCTS (Not Used)

- EXECUTION

      - MAINTENANCE OF PARKING AND ACCESS ROADS

        - Maintenance:  Contractor shall maintain traffic and parking areas in a sound condition.  Contractor shall repair breaks, potholes, low areas, standing water and other deficiencies, to maintain paving and drainage in original or specified condition.

        - Cleaning of Sidewalks, Roadways, and Parking Areas:  Contractor shall keep public and private rights-of-way and parking areas clear of construction-caused soiling, dust and debris, especially debris hazardous to vehicle tires.

          - Contractor shall inspect and perform cleaning hourly to ensure entire public commons areas, sidewalks, crosswalks, roadways, haul routes, and parking lots are free of all debris.  Contractor shall provide dedicated laborers and equipment as required to ensure areas are kept neat and clean during each day of the contract.

          - Contractor shall coordinate with requirements specified in Section 01 57 00 - Temporary Controls and Section 01 74 00 - Cleaning Requirements.

END OF SECTION